# COIT12206 Week 5 Tutorial - VLAN Configuration Report

# Student Details

Name: Elish Niraula
ID: 12313870
Week: 5

# Task 1: VLAN Setup on Switch

In Task 1, four Linux hosts and one Open vSwitch were configured in GNS3.

All hosts were assigned IP addresses in the same subnet:
Host1: 10.10.1.101
Host2: 10.10.1.102
Host3: 10.10.1.103
Host4: 10.10.1.104

Connectivity test using ping showed successful communication between all hosts.

VLAN Configuration:
Based on the student ID (870), VLAN IDs used:
VLAN 870 → Host1 & Host2
VLAN 900 → Host3 & Host4

Switch configuration commands:
ovs-vsctl set port eth1 tag=870
ovs-vsctl set port eth2 tag=870
ovs-vsctl set port eth3 tag=900
ovs-vsctl set port eth4 tag=900

After VLAN configuration, hosts in different VLANs were isolated and could not communicate.
Screenshots of done task 1:

Connectivity of the host and switch.

# Task 2: VLAN Setup with Router

In Task 2, a Linux router was added to enable inter-VLAN communication.

Hosts were divided into two subnets:
VLAN 870 → 10.10.1.0/24
VLAN 900 → 10.10.2.0/24

Switch trunk configuration:
ovs-vsctl set port eth0 trunks=[]

Router configuration:
ip link add link eth0 name eth0.10 type vlan id 10
ip link add link eth0 name eth0.20 type vlan id 20

IP assignment:
ip address add 10.10.1.1/24 dev eth0.10
ip address add 10.10.2.1/24 dev eth0.20

Interfaces were brought up:
ip link set dev eth0.10 up
ip link set dev eth0.20 up

After configuration, all hosts successfully communicated across VLANs via the router.

Screenshots of task 2:

# Results

• Successful VLAN segmentation in Task 1.
• Inter-VLAN routing achieved in Task 2.
• Ping tests confirmed connectivity.

# Reflection

This lab helped in understanding VLAN concepts and their implementation using Open vSwitch. 
Initially, VLANs restricted communication between hosts, demonstrating network segmentation. 
With the addition of a router and trunk configuration, inter-VLAN communication was successfully achieved. 
This practical exercise improved understanding of switching, tagging, trunk ports, and routing between networks.